# Measure 1: Fryers-Commercial-Electric

### Measure Description

Commercial Electric fryers are among the most common pieces of cooking equipment in commercial food service facilities. Recent advances in equipment design have produced fryers that operate more efficiently, quickly, safely and conveniently. Energy efficient commercial fryers reduce energy consumption primarily through the application of controls and insulation.

With the availability of ENERGY STAR® rated models of fryers, it is straightforward to differentiate between high efficiency and standard efficiency models. This measure is focused on 14-inch open deep fat electric fryers with a nominal shortening capacity between 35 and 50 pounds.

Energy efficient fryers that have earned the ENERGY STAR® offer shorter cook times and higher production rates. Frypot insulation reduces standby losses resulting in a lower idle energy rate.

Fryer performance is determined by applying the ASTM Standard Test Method for the Performance of Open Deep Fat Fryers (F1361-05).  The ASTM standard test method is considered the industry standard for quantifying the efficiency and performance of open deep fat fryers.

Market Applicability

This measure is applicable to any small commercial cooking application. Includes (but not limited to) casual dining and quick service restaurants, hotels, motels, schools, colleges and recreational facilities.

# Terms and Conditions:

This incentive applies towards the purchase of new or replacement energy efficient electric fryers (fuel switching applications are not eligible). Used or rebuilt equipment is not eligible. Customers must provide proof that the appliance meets the energy efficiency specifications listed in Table 1.1

Table 1.1 Energy Efficiency Requirements for Commercial Electric Fryers.

| Test Description | Cooking Energy Efficiency |
| --- | --- |
| Heavy Load Cooking Energy Efficiency ASTM F1361 | ≥ 80% |

### Cost Effectiveness Modeling Measure Data

High efficiency electric fryers typically list for more than standard efficiency electric fryers. A typical standard efficiency fryer lists from $1,500 to $2,000. However, high efficiency designs are often bundled with other features such as all stainless steel construction and high quality components and controls. In addition to lower operating costs, high efficiency fryers exhibit higher production rates and shorter recovery times than Baseline model fryers and in some cases may eliminate the need for a backup fryer.

Measure data for cost effectiveness modeling are based on average equipment characteristics for California utility customer participants for the Food Service Equipment program. Unitized cost effectiveness determinants are summarized in Table 1.2. Annual energy use was calculated based on preheat, idle, and cooking energy efficiency and production capacity test results from applying ASTM F1361-05. Annual energy use in this example is based on the fryer operating for 16 hours a day, 365 days per year, with one preheat daily and cooking 150 pounds/day of food. The assumed price for electricity is $0.13 per kWh.

# Table 1.2 Commercial Electric Fryer Cost Effectiveness Example

| Performance | Baseline Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 2.30 | 1.70 |
| Idle Energy Rate (kW) | 1.05 | 1.00 |
| Heavy Load Cooking Energy Efficiency (%) | 75% | 80% |
| Production Capacity (lbs/hr) | 65 | 70 |
| Operating Hours/Day | 16 | 16 |
| Operating Days/Year | 365 | 365 |
| Pounds of Food Cooked per Day | 150 | 150 |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| ASTM Energy to Food (kWh/lb) | 0.167 | 0.167 |
| Daily Energy Consumption (kWh) | 49.8 | 46.6 |
| Average Demand (kW) | 3.1 | 2.9 |
| Estimated Demand Reduction (kW) | - | 0.2 |
| Annual Energy Consumption (kWh) | 18,182 | 17,016 |
| Estimated Energy Savings (kWh/yr) | - | 1,166 |
| Annual Energy Cost ($) | $2,364 | $2,212 |
| Estimated Cost Savings ($/yr) | - | $152 |
| Incremental Measure Cost | - | SEE APPENDIX A |
| Estimated Useful Life (EUL)a | 12 years | 12 years |

a The estimated useful life is based on DEER estimates for food service equipment and filed in the Energy Efficiency Policy Manual Table 4.1.

Daily Energy Consumption Calculation and Definitions

EDAY = LBFOOD x EFOOD ÷ EFFICIENCY + IDLERATE x (TON - LBFOOD/PC – TP/15) + EP

Where:

| EDAY = | Daily Energy Consumption (kWh) |
| --- | --- |
| LBFOOD = | Pounds of Food Cooked per Day |
| EFOOD = | ASTM Energy to Food (kWh/lb) = kWh/pound of energy absorbed by food product during cooking |
| EFFICIENCY = | Heavy Load Cooking Energy Efficiency % |
| IDLE RATE = | Idle Energy Rate (kW) |
| TON = | Operating Hours/Day |
| PC = | Production Capacity (lbs/hr) |
| TP = | Preheat Time (min) |
| EP = | Preheat Energy (kWh) |

Fryers-Commercial-Electric

APPENDIX A

| Make Energy Efficient | Model | Fuel Source | Cost($)* |
| --- | --- | --- | --- |
| Frymaster | H-14 | Electric | $12,160 |
| Frymaster | H17TC | Electric | $11,424 |
| Pitco | SEH50 | Electric | $7,192 |
| Average Cost of Energy Efficient Fryer - Electric |  |  | $10,259 |
| Make Energy Baseline | Model | Fuel Source | Cost($)* |
| Anets | ANE-14EL14AA | Electric | $5,822 |
| Pitco | SE14-SSTC | Electric | $5,280 |
| Average Cost of Baseline Model Fryer - Electric |  |  | $5,551 |
| List Price Average Incremental Cost Difference |  |  | $4,708 |
| *Costs taken from published manufacturers list prices |  |  |  |

Measure 2: Griddles-Commercial-Electric

Measure Description

Commercial electric griddles are used throughout the hospitality industry, typically occupying a central position on the short order line. Its versatility ranges from crisping and browning, to searing, and to warming or toasting. For a high production kitchen, the temperature uniformity of the griddle surface is important to assure that the food is evenly cooked.

Recent advances in griddle design have produced equipment that exhibits greater uniformity, are better controlled, and provide higher production rates. Energy efficient commercial electric griddles reduce energy consumption primarily through application of advanced controls and improved temperature uniformity. This measure is focused on electric “flat” (single sided) griddles.

Griddle performance is determined by applying the ASTM Standard Test Method for the Performance of Griddles (F1275).  The ASTM standard test method is considered to be the industry standard for quantifying the efficiency and performance of griddles.

### Market Applicability

This measure is applicable to any small commercial cooking application. Includes (but not limited to) casual dining and quick service restaurants, hotels, motels, schools, colleges and recreational facilities.

# Terms and Conditions:

This incentive applies towards the purchase of new or replacement energy efficient electric griddles (fuel switching applications are not eligible). Used or rebuilt equipment is not eligible. Customers must provide proof that the appliance meets the energy efficiency specifications listed in Table 2.1

Table 2.1. Energy Efficiency Requirements for Commercial Electric Griddles.

| Test Description | Cooking Energy Efficiency |
| --- | --- |
| Heavy Load Cooking Energy Efficiency ASTM F1275 |  70% |

### Cost Effectiveness Modeling Measure Data

High efficiency electric griddles typically list for more than standard efficiency electric griddles. However, high efficiency designs are often bundled with other features such as all stainless steel construction and high quality components and controls. In addition to lower operating costs, high efficiency griddles exhibit better uniformity and higher production rates than Baseline model griddles. For example, an energy efficient, 3-foot griddle can produce as much as a 4-foot Baseline model.

Measure data for cost effectiveness modeling are based on average equipment characteristics for California utility customer participants for the Food Service Equipment program. Unitized cost effectiveness determinants are summarized in Table 2.2. Annual energy use was calculated based on preheat, idle, and cooking energy efficiency and production capacity test results from applying ASTM F1275. Annual energy use in this example is based on the griddle operating for 12 hours a day, 365 days per year, with one preheat daily and cooking 100 pounds/day of food. The assumed price for electricity is $0.13 per kWh.

Table 2.2. Commercial Electric Griddle Cost Effectiveness Example.

| Performance | Baseline Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 4.00 | 2.00 |
| Idle Energy Rate (kW) | 2.50 | 2.30 |
| Cooking-Energy Efficiency (%) | 65% | 70% |
| Production Capacity (lb/h) | 35 | 40 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| ASTM Energy to Food (kWh/lb) | 0.139 | 0.139 |
| Daily Energy Consumption (kWh) | 47.6 | 43.1 |
| Average Demand (kW) | 4.0 | 3.6 |
| Estimated Demand Reduction (kW) | - | 0.4 |
| Annual Energy Consumption (kWh) | 17,380 | 15,743 |
| Estimated Energy Savings (kWh/yr) | - | 1,637 |
| Annual Energy Cost ($) | $2,259 | $2,047 |
| Estimated Cost Savings ($/yr) | - | $213 |
| Incremental Measure Cost | - | SEE APPENDIX A |
| Estimated Useful Life (EUL)a | 12 years | 12 years |

a The estimated useful life is based on DEER estimates for food service equipment and filed in the Energy Efficiency Policy Manual Table 4.1.

Daily Energy Consumption Calculation and Definitions

EDAY = LBFOOD X EFOOD ÷ EFFICIENCY + IDLERATE x (TON - LBFOOD/PC – TP/60) + EP

Where:

| EDAY = | Daily Energy Consumption (kWh) |
| --- | --- |
| LBFOOD = | Pounds of Food Cooked per Day |
| EFOOD = | ASTM Energy to Food (kWh/lb) = kWh/pound of energy absorbed by food product during cooking |
| EFFICIENCY = | Heavy Load Cooking Energy Efficiency % |
| IDLE RATE = | Idle Energy Rate (kW) |
| TON = | Operating Hours/Day |
| PC = | Production Capacity (lbs/hr) |
| TP = | Preheat Time (min) |
| EP = | Preheat Energy (kWh) |

# Griddles-Commercial-Electric

# APPENDIX A

| Make Energy Efficient | Model | Fuel Source | Cost ($)* |
| --- | --- | --- | --- |
| Wells | WELG23 | Electric | $3,182 |
| Keating | KEA-3030MEG | Electric | $6,094 |
| AccuTemp | EGF2083A36-00 | Electric | $7,190 |
| Average Cost of Energy Efficient Griddle - Electric |  |  | $5,489 |
| Make Baseline | Model | Fuel Source | Cost ($)* |
| Star | STR536TGD | Electric | $1,908 |
| APW Wyott | EG-36H | Electric | $1,862 |
| Average Cost of Baseline Model Griddle - Electric |  |  | $1,885 |
| List Price Average Incremental Cost Difference |  |  | $3,604 |
| *Costs taken from published manufacturers lists price |  |  |  |

# Measure 3: Pressureless Steamers-Commercial-Electric (Connectionless/Boilerless)

Measure Description

Pressureless electric steamers provide a fast cooking option for preparing large quantities of food, while retaining vital nutrients in the cooked product. In addition, steamers can be used to gently rethermalize food products. Steamers come in a variety of configurations, including countertop models, wall mounted models and floor models mounted on a stand, pedestal or cabinet style base. A steamer may consist of one to four stacked cavities, though two compartment steamers are the most prevalent in the industry. The cavity is usually designed to accommodate a standard 12" x 20" hotel pan.

The steam itself can be produced several ways. Many compartment steamers have an external (with respect to the cooking compartment) gas, electric, or service steam powered boiler that produces potable steam under pressure. This pressurized steam is delivered to the cooking compartment as demanded by the control settings. However, in the case of a pressureless steamer, the compartment is openly connected to a condensate drain and the steam environment within the compartment cannot sustain a pressure above atmospheric (both raw steam and condensate exit the cooking cavity through this drain).

Steam also may be produced by a steam generator located within (or directly connected to) the cooking cavity. This method differs from the boiler based steamers in that the steam is produced at (or slightly above) the compartment operating pressure (i.e., atmospheric pressure). This strategy is not used for pressure steamers. A steamer may produce steam by boiling water poured directly into the cooking compartment prior to operation (this is the simplest form of an internal steam generator, typically referred to as a “connectionless” steamer). Electric elements are typically located directly beneath the compartment’s floor.

With the availability of ENERGY STAR® rated models of Electric steamers, it is fairly straightforward to differentiate between high efficiency and standard efficiency models. Steamer performance is determined by applying the ASTM Standard Test Method for the Performance of Steam Cookers (F1484).  The ASTM standard test method is considered to be the industry standard for quantifying the efficiency and performance of steamers.

### Market Applicability

This measure is applicable to any small commercial cooking application. Includes (but not limited to) casual dining and quick service restaurants, hotels, motels, schools, colleges and recreational facilities.

# Terms and Conditions:

This incentive applies towards the purchase of new or replacement energy efficient electric steamers (fuel switching applications are not eligible). Used or rebuilt equipment is not eligible. Customers must provide proof that the appliance meets the energy efficiency specifications listed in Table 3.1.

Table 3.1 Energy Efficiency Requirements for Electric Pressureless Steamers.

| Test Description | Cooking Energy Efficiency |
| --- | --- |
| Heavy Load (Potato) Cooking Energy Efficiency ASTM F1484 | ≥50% |

### Cost Effectiveness Modeling Measure Data

Measure data for cost effectiveness modeling are based on average equipment characteristics for California utility customer participants for the Food Service Equipment program. Unitized cost effectiveness determinants are summarized in Table 3.2. Annual energy use was calculated based on preheat, idle, and potato cooking energy efficiency and production capacity test results from applying ASTM F1484. Annual energy use in this example is based on the steamer operating for 12 hours a day, 365 days per year, with one preheat daily and cooking 100 pounds/day of food. The assumed electricity price is $.13 per kWh and the assumed combined cost for water and sewer is $5/CCF.

# Table 3.2 Pressureless Electric Steamer Cost Effectiveness Example

| Performance | Baseline Model | Energy Efficient Model |
| --- | --- | --- |
| Pan Capacity | 3 | 3 |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 1.50 | 1.50 |
| Idle Energy Rate (kW) | 1.00 | 0.40 |
| Cooking-Energy Efficiency (%) | 26% | 50% |
| Production Capacity (lb/h) | 70 | 50 |
| Average Water Consumption Rate (gal/h) | 40 | 3 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| Water/Sewer Cost ($/CCF) | $5.00 | $5.00 |
| ASTM Energy to Food (kWh/lb) | 0.0308 | 0.0308 |
| Residual Energy Rate (kW) | 1.91 | 0.12 |
| Daily Energy Consumption (kWh) | 43.4 | 12.8 |
| Average Demand (kW) | 3.6 | 1.1 |
| Estimated Demand Reduction (kW) | - | 2.5 |
| Annual Energy Consumption (kWh) | 15,824 | 4,658 |
| Estimated Energy Savings (kWh/yr) | - | 11,166 |
| Annual Water Consumption (gal) | 175,200 | 13,140 |
| Estimated Water Savings (gal) | - | 162,060 |
| Annual Energy Cost ($) | $2,057 | $606 |
| Estimated Cost Savings ($/yr) | - | $1,452 |
| Annual Water Cost ($/yr) | $1,171 | $88 |
| Estimated Water Cost Savings ($/yr)a | - | $1,083 |
| Incremental Measure Cost | - | SEE APPENDIX A |
| Estimated Useful Life (EUL)b | 12 years | 12 years |

a  Water and wastewater cost are based on a rate of $2.00/CCF water and $3.00/CCF wastewater.

(1 CCF = 748 gallons)

b The estimated useful life is based on DEER estimates for food service equipment and filed in the Energy Efficiency Policy Manual Table 4.1.

Daily Energy Consumption Calculation and Definitions

EDAY = LBFOOD x EFOOD ÷ EFFICIENCY + IDLERATE x (TON - LBSFOOD/PC – TP/60) + RESIDUALRATE x (TON - LBSFOOD ÷ PC - TP)+ EP

Where:

| EDAY = | Daily Energy Consumption (kWh) |
| --- | --- |
| LBFOOD = | Pounds of Food Cooked per Day |
| EFOOD = | ASTM Energy to Food (kWh/lb) = kWh/pound of energy absorbed by food product during cooking |
| EFFICIENCY = | Heavy Load Cooking Energy Efficiency % |
| IDLE RATE = | Idle Energy Rate (kW) |
| TON = | Operating Hours/Day |
| PC = | Production Capacity (lbs/hr) |
| TP = | Preheat Time (min) |
| EP = | Preheat Energy (kWh) |
| RESIDUALRATE = | Residual Energy Rate (kW) |

Pressureless Steamers-Commercial-Electric (Connectionless/Boilerless)

APPENDIX A

| Make Energy Efficient | Model | Fuel Source | Cost($)* |
| --- | --- | --- | --- |
| AccuTemp | 208D12 | Electric | $9,597 |
| AccuTemp | 208D12 | Electric | $9,597 |
| AccuTemp | 208D6 | Electric | $9,199 |
| AccuTemp | 208D6 | Electric | $9,199 |
| AccuTemp | 208D8 | Electric | $9,597 |
| AccuTemp | 208D8 | Electric | $9,597 |
| AccuTemp | 240D12 | Electric | $9,597 |
| AccuTemp | 240D12 | Electric | $9,597 |
| AccuTemp | 240D6 | Electric | $9,199 |
| AccuTemp | 240D6 | Electric | $9,199 |
| AccuTemp | 240D8 | Electric | $9,597 |
| AccuTemp | 240D8 | Electric | $9,597 |
| Groen | VRC-6E | Electric | $9,202 |
| Market Forge | ET-3E | Electric | $6,975 |
| Market Forge | ET-6E | Electric | $8,758 |
| Market Forge | STP-6E | Electric | $22,282 |
| Vulcan | PX-3 | Electric | $6,630 |
| Vulcan-Hart | PX-5 | Electric | $9,290 |
| Average Cost of Energy Efficient Steamer - Electric |  |  | $9,817 |
| Make Baseline | Model | Fuel Source | Cost($)* |
| Southbend | R2 | Electric | $5,395 |
| Cleveland | 21CET82083MCS | Electric | $5,940 |
| Average Cost of Baseline Model Steamer - Electric |  |  | $5,668 |
| List Price Average Incremental Cost Difference |  |  | $4,150 |
| *Costs taken from published manufacturers list prices |  |  |  |

# Measure 4: Convection Oven-Commercial-Electric

Measure Description

Commercial electric convection ovens are the most widely used appliances in the food service industry. Many food service operations rely heavily on the versatility of ovens. Operators can cook varieties of foods in large quantities with a single appliance. An oven can be simply described as a fully enclosed, insulated chamber used to heat food. With competition rising among equipment manufacturers, new designs that incorporate timesaving features via sophisticated control packages are being introduced.

Ovens represent the largest appliance category in terms of the types of units manufactured of any of the major cooking equipment categories. This versatility and diversity mean that they can be found in almost any type of food service operation. A recent US study showed that 95% of commercial (non-institutional) operations reported using at least one type of oven; 98% of noncommercial (institutional) operations reported the same. The percentage of operations, commercial and institutional, using general bake ovens was 52% and 56%, respectively. Fifty percent of the operations in the commercial sector reported using convection ovens as compared to 83% of noncommercial operations.

Oven performance is determined by applying the ASTM Standard Test Method for the Performance of Convection Ovens (F1496).  The ASTM standard test method is considered to be the industry standard for quantifying the efficiency and performance of convection ovens. This measure is focused on standard full size and half size electric convection ovens.

### Market Applicability

This measure is applicable to any small commercial cooking application. Includes (but not limited to) casual dining and quick service restaurants, hotels, motels, schools, colleges and recreational facilities.

### Terms and Conditions

This incentive applies towards the purchase of new or replacement energy efficient electric convection ovens (fuel switching applications are not eligible). Customers must provide proof that the appliance meets the energy efficiency specifications listed in Table 4.1.

Table 4.1. Energy Efficiency Requirements for Electric Convection Ovens.

| Test Description | Cooking Energy Efficiency |
| --- | --- |
| Heavy Load (Potato) Cooking Energy Efficiency ASTM F1496 |  70% |

### Cost Effectiveness Modeling Measure Data

High efficiency electric ovens typically list for more than standard efficiency electric ovens. However, high efficiency designs are often bundled with other features such as all stainless steel construction and high quality components and controls. In addition to lower operating costs, high efficiency ovens frequently exhibit better baking uniformity.

Measure data for cost effectiveness modeling are based on average equipment characteristics for California utility customer participants for the Food Service Equipment program. Unitized cost effectiveness determinants are summarized in Table 4.2. Annual energy use was calculated based on preheat, idle, and cooking energy efficiency and production capacity test results from applying ASTM F1496. Annual energy use in this example is based on the oven operating for 12 hours a day, 365 days per year, with one preheat daily and cooking 100 pounds/day of food. The assumed electric price is $0.13 per kWh.

Table 4.2 Electric Convection Oven Cost Effectiveness Example.

| Performance | Baseline Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 1.50 | 1.00 |
| Idle Energy Rate (kW) | 2.00 | 1.50 |
| Heavy Load Cooking Energy Efficiency (%) | 65% | 70% |
| Production Capacity (lb/h) | 70 | 80 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| ASTM Energy to Food (kWh/lb) | 0.0732 | 0.0732 |
| Daily Energy Consumption (kWh) | 33.4 | 27.2 |
| Average Demand (kW) | 2.8 | 2.3 |
| Estimated Demand Reduction (kW) | - | 0.5 |
| Annual Energy Consumption (kWh) | 12,193 | 9,931 |
| Estimated Energy Savings (kWh/yr) | - | 2,262 |
| Annual Energy Cost ($) | $1,585 | $1,291 |
| Estimated Cost Savings ($/yr) | - | $294 |
| Incremental Measure Cost | - | SEE APPENDIX A |
| Estimated Useful Life (EUL)a | 12 years | 12 years |

a The estimated useful life is based on DEER estimates for food service equipment and filed in the Energy Efficiency Policy Manual Table 4.1.

Daily Energy Consumption Calculation and Definitions

EDAY = LBFOOD x EFOOD ÷ EFFICIENCY + IDLERATE x (TON - LBFOOD/PC – TP/60) + EP

Where:

| EDAY = | Daily Energy Consumption (kWh) |
| --- | --- |
| LBFOOD = | Pounds of Food Cooked per Day |
| EFOOD = | ASTM Energy to Food (kWh/lb) = kWh/pound of energy absorbed by food product during cooking |
| EFFICIENCY = | Heavy Load Cooking Energy Efficiency % |
| IDLE RATE = | Idle Energy Rate (kW) |
| TON = | Operating Hours/Day |
| PC = | Production Capacity (lbs/hr) |
| TP = | Preheat Time (min) |
| EP = | Preheat Energy (kWh) |

Convection Oven-Commercial-Electric

APPENDIX A

| Make Energy Efficient | Model | Fuel Source | Cost ($)* |
| --- | --- | --- | --- |
| Vulcan | VC4ED | Electric | $8,020 |
| Wolf Range | WOLWKED1 | Electric | $7,619 |
| Average Cost of Energy Efficient Convection Oven |  |  | $7,820 |
| Make Baseline | Model | Fuel Source | Cost ($)* |
| Duke | E-101E | Electric | $4,934 |
| Baker's Pride | BAKBCOE1 | Electric | $5,280 |
| Average Cost of Baseline Model Convection Oven |  |  | $5,107 |
| List Price Average Incremental Cost Difference |  |  | $2,713 |
| *Costs taken from published manufacturers lists price |  |  |  |

Measure 5: Combination Oven-Commercial-Electric

Measure Description

An oven can be simply described as a fully enclosed, insulated chamber used to heat food. Commercial electric combination ovens offer even more options with their ability to add steam to the oven cavity. In addition to baking and roasting, a combination oven is also capable of steaming, proofing and rethermalizing various food products. Foods can be cooked in a convection oven dry heat only mode, a steam only mode, and a combination of dry heat and steam modes. The programmability of combination ovens also allows food to be cooked partially in one mode at a certain temperature, and then finished in another mode and at a separate temperature. For example, a turkey can be cooked in combination mode at low temperature for several hours, and then stepped to a higher temperature in dry heat mode to finish.

Fifty percent of the operations in the commercial sector reported using combination ovens as compared to 83% of noncommercial operations. With competition rising among equipment manufacturers, new designs that incorporate timesaving features via sophisticated control packages are being introduced.

Oven performance is determined by applying the ASTM Standard Test Method for the Performance of Combination Ovens (F1639-05).  The ASTM standard test method is considered to be the industry standard for quantifying the efficiency and performance of combination ovens. This measure is focused on electric combination ovens.

### Market Applicability

This measure is applicable to any small commercial cooking application. Includes (but not limited to) casual dining and quick service restaurants, hotels, motels, schools, colleges and recreational facilities.

### Terms and Conditions

This incentive applies towards the purchase of new or replacement energy efficient electric combination ovens (fuel switching applications are not eligible). Customers must provide proof that the appliance meets the energy efficiency specifications listed in Table 5.1.

Table 5.1. Energy Efficiency Requirements for Electric Combination Ovens.

| Test Description | Cooking Energy Efficiency |
| --- | --- |
| Heavy Load Cooking Energy Efficiency ASTM F1639-05 |  60% |

### Cost Effectiveness Modeling Measure Data

High efficiency electric combination ovens typically list for more than standard-efficiency electric combination ovens. However, high-efficiency designs are often bundled with other features such as all-stainless steel construction and high-quality components and controls. In addition to lower operating costs, high-efficiency combination ovens frequently exhibit higher production capacities.

Measure data for cost effectiveness modeling have been developed based on average equipment characteristics for California utility customer participants for the Food Service Equipment program. Unitized cost effectiveness determinants are summarized in Table 5.2. Annual energy use was calculated based on preheat, idle, and cooking-energy efficiency and production capacity test results from applying ASTM F1639-05. Annual energy use in this example is based on standard 10-pan oven operating for 12 hours a day, 365 days per year, and cooking 200 pounds per day of food. The assumed electric price is $0.13 per kWh.

Table 5.2. Electric Combination Oven Cost Effectiveness Example.

| Performance | Baseline Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 3.00 | 1.50 |
| Idle Energy Rate (kW) | 7.50 | 3.00 |
| Cooking-Energy Efficiency (%) | 44% | 60% |
| Production Capacity (lb/h) | 80 | 100 |
| Average Water Consumption Rate (gal/h) | 40 | 20 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Pounds of Food Cooked per Day | 200 | 200 |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| Water/Sewer Cost ($/CCF) | $5.00 | $5.00 |
| ASTM Energy to Food (kWh/lb) | 0.0732 | 0.0732 |
| Daily Energy Consumption (kWh) | 105.6 | 55.2 |
| Average Demand (kW) | 8.8 | 4.6 |
| Estimated Demand Reduction (kW) | - | 4.2 |
| Annual Energy Consumption (kWh) | 38,561 | 20,130 |
| Estimated Energy Savings (kWh/yr) | - | 18,432 |
| Annual Water Consumption (gal) | 175,200 | 87,600 |
| Estimated Water Savings (gal) | - | 87,600 |
| Annual Energy Cost ($) | $5,013 | $2,617 |
| Estimated Cost Savings ($/yr) | - | $2,396 |
| Annual Water Cost ($/yr)a | $1,171 | $586 |
| Estimated Water Cost Savings ($/yr) | - | $586 |
| Incremental Measure Cost | - | SEE APPENDIX A |
| Estimated Useful Life (EUL)b | 12 years | 12 years |

a  Water and wastewater cost are based on a rate of $2.00/CCF water and $3.00/CCF wastewater.

(1 CCF = 748 gallons)

b The estimated useful life is based on DEER estimates for food service equipment and filed in the Energy Efficiency Policy Manual Table 4.1.

Daily Energy Consumption Calculation and Definitions

EDAY = LBFOOD x EFOOD ÷ EFFICIENCY + IDLERATE x (TON - LBFOOD/PC – TP/60) + EP

Where:

| EDAY = | Daily Energy Consumption (kWh) |
| --- | --- |
| LBFOOD = | Pounds of Food Cooked per Day |
| EFOOD = | ASTM Energy to Food (kWh/lb) = kWh/pound of energy absorbed by food product during cooking |
| EFFICIENCY = | Heavy Load Cooking Energy Efficiency % |
| IDLE RATE = | Idle Energy Rate (kW) |
| TON = | Operating Hours/Day |
| PC = | Production Capacity (lbs/hr) |
| TP = | Preheat Time (min) |
| EP = | Preheat Energy (kWh) |

Combination Oven-Commercial-Electric

APPENDIX A

| Make Energy Efficient | Model | Fuel Source | Cost ($)* |
| --- | --- | --- | --- |
| Rational/ Henny Penny | SCC62 | Electric | $24,100 |
| Average Cost of Energy Efficient Combination Oven |  |  | $24,100 |
| Make Baseline | Model | Fuel Source | Cost ($)* |
| Alto Shaam | 605ML/STD | Electric | $8,004 |
| Garland Moisture Plus | MP-ES10 D | Electric | $6,428 |
| Average Cost of Baseline Model Combination Oven |  |  | $7,216 |
| List Price Average Incremental Cost Difference |  |  | $16,884 |
| *Costs taken from published manufacturers lists price |  |  |  |

# Measure 6: Insulated Hot Food Holding Cabinets-Commercial

Measure Description

Commercial insulated hot food holding cabinet models that meet program requirements incorporate better insulation, reducing heat loss, and may also offer additional energy saving devices such as magnetic door electric gaskets, auto-door closures, or dutch doors. The insulation of the cabinet also offers better temperature uniformity within the cabinet from top to bottom. This means that qualified hot food holding cabinets are more efficient at maintaining food temperature while using less energy.

Market Applicability

This measure is applicable to any small commercial cooking application. Includes (but not limited to) casual dining and quick service restaurants, hotels, motels, schools colleges and recreational facilities.

# Terms and Conditions:

This incentive applies towards the purchase of new or replacement energy efficient commercial electric Hot Food Holding Cabinets. This measure does not include cook and hold equipment. All measures must be electric hot food holding cabinets that are fully insulated and have solid doors, in full, three-quarter, and half sizes respectively. Used or rebuilt equipment is not eligible. Customers must provide proof that the appliance meets the energy efficiency specifications listed in Table 6.1.

Table 6.1 Energy Efficiency Requirements for Hot Food Holding Cabinets

| Description | Qualifying Operating Energy Rate |
| --- | --- |
| For full size cabinets | Maximum Idle Energy Rate of 20 Watts/ft3 |
| For three-quarter size cabinets | Maximum Idle Energy Rate of 20 Watts/ft3 |
| For half size cabinets | Maximum Idle Energy Rate of 20 Watts/ft3 |

Cost Effectiveness Modeling Measure Data

Energy usage calculations are based on 15 hours a day, 365 days per year operation at a typical temperature setting of 150°F. Note that the different sizes for the holding cabinets (half size and three-quarter size) have proportional operating energy rates. Operating energy rate for the full size holding cabinets was obtained in accordance with the American Society for Testing and Materials (ASTM) Standard F2140. Electrical energy cost is estimated at $0.13 per kWh.

The California Energy Commission (CEC) Title 20 code effective January 1, 2006 requires all hot food holding cabinets sold in California meet the ENERGY STAR® specifications for energy efficiency. The energy savings calculations listed in Tables 6.2.1, 6.2.2, and 6.2.3 use the ENERGY STAR® specification as the baseline for potential energy savings. The ENERGY STAR® specification is based on Maximum Idle Energy Rate (MIER) of 40 Watts/ft3.

Table 6.2.1 Insulated Hot Food Holding Cabinet Cost Effectiveness Example

# Full Size Holding Cabinets

| # Performance | # Baseline (ENERGY STAR®) | # Qualifying Model MIER 20 Watt ft3 |
| --- | --- | --- |
| Operating Energy Rate | 0.8 kW | 0.4 kW |
| Annual Energy Use | 4,380 kWh | 2,190 kWh |
| # Annual Energy Costa | # $569 | $285 |
| Coincident Peak Reduction | - | 0.4 kW |
| Annual Energy Savings | - | 2,190 kWh |
| Annual Energy Cost Savings | - | $285 |
| Incremental Measure Cost | - | SEE APPENDIX A |
| Estimated Useful Life (EUL)a | 12 | 12 |

# Table 6.2.2 Insulated Hot Food Holding Cabinet Cost Effectiveness Example

# Three-Quarter Size Holding Cabinets

| # Performance | # Baseline (ENERGY STAR®) | # Qualifying Model MIER 20 Watt ft3 |
| --- | --- | --- |
| Operating Energy Rate | 0.6 kW | 0.3 kW |
| Annual Energy Use | 3,285 kWh | 1,643 kWh |
| # Annual Energy Costa | # $427 | $214 |
| Coincident Peak Reduction | - | 0.3 kW |
| Annual Energy Savings | - | 1,643 kWh |
| Annual Energy Cost Savings | - | $214 |
| Incremental Measure Cost | - | SEE APPENDIX A |
| Estimated Useful Life (EUL)a | 12 | 12 |

# Table 6.2.3 Insulated Hot Food Holding Cabinet Cost Effectiveness Example

#### Half Size Holding Cabinets

| # Performance | # Baseline (ENERGY STAR®) | # Qualifying Model MIER 20 Watt ft3 |
| --- | --- | --- |
| Operating Energy Rate | 0.4 kW | 0.2 kW |
| Annual Energy Use | 2,190 kWh | 1,095 kWh |
| # Annual Energy Costa | # $285 | $142 |
| Coincident Peak Reduction | - | 0.2 kW |
| Annual Energy Savings | - | 1,095 kWh |
| Annual Energy Cost Savings | - | $142 |
| Incremental Measure Cost | - | SEE APPENDIX A |
| Estimated Useful Life (EUL)a | 12 | 12 |

# a The estimated useful life is based on DEER estimates for food service equipment and filed in the Energy Efficiency Policy Manual Table 4.1.

Insulated Hot Food Holding Cabinets-Commercial

APPENDIX A

| Make Energy Efficient (< 20 Watts/ft3) | Model | Fuel Source | Cost($)* |
| --- | --- | --- | --- |
| Carter-Hoffmann | BR96 | Electric | $9,006 |
| Cres Cor | CCB-120A | Electric | $8,980 |
| Cres Cor | CCB-150 | Electric | $9,584 |
| Cres Cor | EB-150 | Electric | $5,722 |
| Cres Cor | H137S96BC | Electric | $5,670 |
| Cres Cor | H137WSUA12C | Electric | $6,178 |
| Cres Cor | H161FUA11CM | Electric | $8,446 |
| Cres Cor | H339SS12188C | Electric | $3,938 |
| FEW | MTU-12 | Electric | $4,884 |
| Average Cost of Energy Efficient Hot Food Holding Cabinet |  |  | $6,934 |
| Make Baseline ENERGY STAR® (< 40 Watts/ft3) | Model | Fuel Source | Cost($)* |
| Cres Cor | H137WSUA5C | Electric | $4,000 |
| Hatco Corporation | FSHC-6W1 | Electric | $3,434 |
| Hatco Corporation | FSHC-7 | Electric | $3,434 |
| Henny Penny | HHC-900 | Electric | $5,279 |
| InterMetro Industries | C190 | Electric | $2,517 |
| InterMetro Industries | MBQ-72 | Electric | $4,025 |
| Average Cost of Baseline Model Hot Food Holding Cabinet |  |  | $3,782 |
| List Price Average Incremental Cost Difference |  |  | $3,153 |
| *Costs taken from published manufacturers lists price |  |  |  |

# Measure 7: Ice Machine-Commercial-Air Cooled

Measure Description

Commercial icemakers represent approximately 11 percent of all commercial refrigeration energy use. The average annual energy use of a 500 lb./day air-cooled icemaker is 5,000 kWh with a potential increase in efficiency of 15 percent (less than a two-year payback). Approximately 64 percent of the commercial ice machine market is self-contained cube making units; the rest are ice making heads and remote condensing units. The PG&E Food Service Technology Center (FSTC) estimates that there are more than 200,000 ice machines in California, with up to 100,000 in PG&E service territory.

Icemakers use a substantial amount of energy in order to freeze water and maintain the ice as separate cubes. Reductions are possible with the use of high efficiency motors in condenser fans and compressors, thicker insulation and reduced evaporator thermal cycling. The energy use in a commercial icemaker varies from product to product, depending on the condenser and the type of ice produced.

Market Applicability

Hospitals account for 39.4 percent of all commercial icemaker purchases, followed by hotels (22.3 percent), restaurants (13.8 percent), retail outlets (8.5 percent), schools (8.5 percent), offices (4.3 percent), and grocery stores (3.2 percent)8.

# Terms and Conditions:

This incentive applies towards the purchase of new or replacement energy efficient Air-cooled ice machines. Used or rebuilt equipment is not eligible. Customers must provide proof that the appliance meets the energy efficiency specifications listed in Table 7.1.

This specification covers machines generating 60 grams (2 oz.) or lighter ice cubes, as well as flaked, crushed, or fragmented ice machines that meet the energy efficiency thresholds by Ice harvest (IHR) rate listed in Table 7.1. Only air cooled machines (icemaker heads, self-contained unites, and remote condensing units) are eligible for incentives. The efficiency specification is an adapted version of the Federal Energy Management Program (FEMP) specification, approximating the top 25 percent of products on the market, based on energy performance.

Performance data is based on ARI Standard 810.

Table 7.1 Energy Efficiency Requirements for Commercial Air Cooled Ice Machines

| Product Type | Ice Harvest Rate (lbs per 24 hrs.)* | Incentive Threshold kWh/100 lbs ice (or Less)** |
| --- | --- | --- |
| Air Cooled | 101-200 | 9.4 |
| Air Cooled | 201-300 | 8.5 |
| Air Cooled | 301-400 | 7.2 |
| Air Cooled | 401-500 | 6.1 |
| Air Cooled | 501-1000 | 5.8 |
| Air Cooled | 1001-1500 | 5.5 |
| Air Cooled | > 1500 | 5.1 |

*   The ice harvest rate (IHR or capacity in lbs) is the amount of ice produced in 24 hours.

** Based on FEMP recommendation for ice maker head

### Cost Effectiveness Modeling Measure Data

In an analysis of the range of products currently on the market, ACEEE research indicates that the highest performing models on the market are 18.46 percent more efficient than the worst performing models, with a payback period of 1.1 years or less. Manufacturers have also indicated that they have the technical capability to exceed current efficiency levels through the use of high efficiency compressors and fan motors, thicker insulation, and other measures. 
First cost estimates assume a 50 percent discount from list price; payback calculations assume annual energy consumption and cost of operating at 75% of rated capacity. Calculations assume an electricity rate of $0.13 per kWh.

# Table 7.2 Commercial Air Cooled Ice Machine Cost Effectiveness Example

| Performance | IHR | IHR | IHR | IHR | IHR | IHR | IHR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ice Harvest Rate (IHR) (lbs per 24 hrs.) | 101-200 | 201-300 | 301-400 | 401-500 | 501-1,000 | 1,001-1,500 | > 1,500 |
| Baselinea (kWh/100 Lbs) | 16 | 11 | 8.5 | 7.6 | 6.9 | 6.4 | 6.1 |
| Incentive Thresholdb(kWh/100 Lbs) | 9.4 | 8.5 | 7.2 | 6.1 | 5.8 | 5.5 | 5.1 |
| Estimated Annual Energy for Baselinec (kWh) | 8,760 | 10,038 | 10,859 | 12,483 | 18,889 | 29,200 | 26,720 |
| Estimated Annual Energy for EE Unitc (kWh) | 5,147 | 7,756 | 9,198 | 10,019 | 15,878 | 25,094 | 22,340 |
| Probable Demand Reductiond (kW) | 0.41 | 0.26 | 0.19 | 0.28 | 0.34 | 0.47 | 0.50 |
| Estimated Annual Energy Savingsc (kWh) | 3,614 | 2,281 | 1,661 | 2,464 | 3,011 | 4,106 | 4,380 |
| Estimated Annual Energy Cost Savings | $470 | $297 | $216 | $320 | $391 | $534 | $569 |
| Incremental Measure Cost | SEE APPENDIX A |  |  |  |  |  |  |
| Estimated Useful Life (EUL) e | 12 | 12 | 12 | 12 | 12 | 12 | 12 |

a Based on FSTC review of ARI Directory and estimate of lower 25 percentile and representation of installed base.

b Based on FEMP recommendation for icemaker head (equivalent to CEE Tier I).

c Assumes ice machine operates at 75% of capacity and median ice harvest rate.

d It has been assumed that the appliance’s probable contribution to the building’s peak demand is the average demand for the appliance.

This is useful because the probability of an appliance drawing its average rate during the period that the building peak is set is significantly higher than for any other input rate for that appliance.

e The estimated useful life is based on DEER estimates for food service equipment and filed in the Energy Efficiency Policy Manual Table 4.1.

Ice Machine-Commercial-Air Cooled

APPENDIX A

| Make Energy Efficient | Model | Ice Harvest Rate (lb/24h)* | Cost($)* |
| --- | --- | --- | --- |
| List Price Estimate |  | 100 | $3,328 |
| Manitowoc | SD322A | 250 | $3,512 |
| Manitowoc | SD0502A | 400 | $3,954 |
| List Price Estimate |  | 450 | $4,721 |
| Manitowoc | SY0604A | 650 | $6,073 |
| Manitowoc | SD1002A | 1060 | $8,225 |
| ICE-O-Matic | ICE-1006HA | 1109 | $8,730 |
| Hoshizaki | KM1300SAH | 1296 | $8,980 |
| Manitowoc | SY1404A | 1450 | $9,462 |
| ICE-O-Matic | ICE-1406HA | 1466 | $9,260 |
| Manitowoc | SD1892N | 1880 | $10,377 |
| Average Cost of 100-200 lb Energy Efficient Ice Machine |  |  | $3,328 |
| Average Cost of 201-300 lb Energy Efficient Ice Machine |  |  | $3,512 |
| Average Cost of 301-400 lb Energy Efficient Ice Machine |  |  | $3,954 |
| Average Cost of 401-500 lb Energy Efficient Ice Machine |  |  | $4,721 |
| Average Cost of 501-1000 lb Energy Efficient Ice Machine |  |  | $7,149 |
| Average Cost of 1001-1500 lb Energy Efficient Ice Machine |  |  | $8,931 |
| Average Cost of >1500 lb Energy Efficient Ice Machine |  |  | $10,377 |
| Make Baseline | Model | Ice Harvest Rate (lb/24h)* | Cost($)* |
| Hoshizaki | AM150 | 150 | $3,070 |
| Manitowoc | ICE-150HA | 171 | $2,995 |
| ICE-O-Matic | ICE-220HA | 238 | $3,200 |
| Scotsman | CME306AS-1C | 340 | $3,395 |
| Scotsman | CME456AS-1C | 480 | $3,720 |
| Scotsman | CME506AS-1F | 500 | $3,760 |
| Hoshizaki | KM630MAH | 619 | $5,640 |
| ICE-O-Matic | ICE-500HA | 625 | $5,125 |
| ICE-O-Matic | ICE-606HA | 706 | $5,875 |
| Scotsman | CME806AS-32F | 770 | $5,850 |
| ICE-O-Matic | ICE-0806FA | 950 | $5,830 |
| Hoshizaki | KM1300MAH | 1040 | $7,110 |
| Cornelius | XAC1444-3PH | 1537 | $7,406 |
| Scotsman | CME1656AS-32F | 1600 | $8,960 |
| Average Cost of 100-200 lb Baseline Model Ice Machine |  |  | $3,033 |
| Average Cost of 201-300 lb Baseline Model Ice Machine |  |  | $3,200 |
| Average Cost of 301-400 lb Baseline Model Ice Machine |  |  | $3,395 |
| Average Cost of 401-500 lb Baseline Model Ice Machine |  |  | $3,740 |
| Average Cost of 501-1000 lb Baseline Model Ice Machine |  |  | $5,664 |
| Average Cost of 1001-1500 lb Baseline Model Ice Machine |  |  | $7,110 |
| Average Cost of >1500 lb Baseline Model Ice Machine |  |  | $8,183 |
| 100-200 lb ice machine** List Price Average Incremental Cost Difference |  |  | $296 |
| 201-300 lb ice machine List Price Average Incremental Cost Difference |  |  | $312 |
| 301-400 lb ice machine List Price Average Incremental Cost Difference |  |  | $559 |
| 401-500 lb ice machine** List Price Average Incremental Cost Difference |  |  | $981 |
| 501-1000 lb ice machine List Price Average Incremental Cost Difference |  |  | $1,485 |
| 1001-1500 lb ice machine List Price Average Incremental Cost Difference |  |  | $1,821 |
| >1500 lb ice machine List Price Average Incremental Cost Difference |  |  | $2,194 |
| *Costs taken from published manufacturers literature |  |  |  |
| **List Price was not available for Energy Efficient Product in the 101-200lb and 401-500lb range. A ratio of baseline case to IMC for the next largest range was applied to baseline case cost of this configuration as an IMC estimate. |  |  |  |

# Measure 8:  Solid Door Reach-In Refrigerator-Commercial

# Solid Door Reach-In Freezers-Commercial

# Glass Door Reach-In Refrigerators-Commercial

Measure Description: Commercial Solid Door Reach-in Refrigerators and Freezers

A reach-in refrigerator is defined as an upright, refrigerated case with solid doors that holds refrigerated products. A reach-in freezer is an upright, refrigerated case with solid doors that holds frozen food products. In both units, the refrigeration system is built in (packaged) and heat is rejected from the refrigeration cycle to the building interior air. Reach-in refrigerators and freezers are used in the food service industry by restaurants and institutional organizations, accounting for approximately 17 percent of all commercial refrigeration energy use. In 1996, the national inventory of refrigerators and freezers in place was 2.1 million units. Estimated annual sales were 233,000 units, 60 percent of which were reach-in refrigerators; the rest were reach-in freezers. The FSTC estimates that there are more than 300,000 reach-in refrigerators and freezers in California, with up to 150,000 in PG&E service area. Approximately 70 percent of these new units are replacing older units. About 30 percent are sold for use in the new construction market. The most common solid-door, reach-in freezer is a one-door unit. Two-door units are also used extensively. The most common solid-door, reach-in refrigerator is a two-door unit; one-door units are used about half as often. The average capacity of a one-door unit is 24 cubic feet and the average capacity of a two-door unit is 48 cubic feet.

The ENERGY STAR® voluntary specification for solid door reach-in refrigerators and freezers was launched in 2001, it identified the top 25 percent (approximately) of products on the market, based on their energy performance. With the influx of new products into the market, this percentage has increased. CEE (Consortium for Energy Efficiency, Inc.) launched an initiative for commercial reach-in, solid-door commercial refrigerators and freezers in December of 2002. The CEE initiative seeks to maximize the energy savings opportunity available through increased and sustained market share of efficient commercial refrigeration equipment. As of January 1, 2006 California Title 20 Code requires that solid door reach-in refrigerators and freezer be manufactured at the ENERGY STAR® or CEE Tier I specification. CEE's specification has two Tiers; Tier I is the equivalent of the ENERGY STAR® specification, Tier II is 40 percent more efficient than Tier I for refrigerators and 30 percent more efficient for freezers. The most common technologies for improving efficiency include; efficient compressors, efficient evaporator fans, efficient condenser fans, anti sweat heaters, electric defrost, increased levels of insulation, and more efficient lighting.

Measure Description: Commercial Glass Door Reach-in Refrigerators

CEE launched an initiative for high-efficiency commercial glass-door, reach-in refrigerators in December of 2003. Commercial glass-door, reach-ins account for about 8 percent of packaged commercial refrigeration energy consumption. Nationally, an estimated 800,000 units are in place, using 4.7 billion kWh annually. Approximately 90,000 units are sold each year.

An estimated 75-90 percent of these units are sold to bottling companies, such as Coca-Cola Bottling Companies and Pepsi Bottling Group. The bottling companies then place units on various sites, such as grocery stores, retail stores and gas stations. Beverage companies have specific requirements for this equipment, such as lighting, ease of loading and rapid cooling of beverages.

CEE's specification has two Tiers of energy efficiency, Tier I is based on the top 25% of products in the market while Tier II is 28 percent more efficient than Tier I. Program incentives and savings calculations are be based on a two tiered approach in 2006 moving toward a CEE Tier II only specification on January 1, 2007 when California title 20 mandates ENERGY STAR® or CEE Tier I as the minimum code requirement.

Market Applicability

Restaurants account for about 80 percent of the solid door reach-in refrigerator and freezer purchases. The remaining purchasers of this equipment are mostly institutional (schools, churches, prisons, etc.) An estimated 75-90 percent of glass door reach-in refrigerator units are sold to bottling companies, such as Coca-Cola Bottling Companies and Pepsi Bottling Group and are used in a variety of retail merchandising end uses. The balance of Glass-door, reach-in units not used for selling beverages are used in commercial restaurant kitchens,  hotels, or schools.

Terms and Conditions:

This incentive applies towards the purchase of new or replacement energy efficient commercial reach-in solid door refrigerators, freezers and glass door reach-in refrigerators. In all categories, the refrigeration system shall be built-in (packaged), cases with remote refrigeration systems do not qualify. Used or rebuilt equipment is not eligible. Customers must provide proof that the appliance meets the energy efficiency specifications listed in Table 8.1. These specifications are also available at the www.cee1.org Web site.

Table 8.1 Commercial Solid Door Reach-In Refrigerators and Freezers and Glass Door Reach-In Refrigerators.

| Measure Description | Qualifying Criteria (Energy Consumption Under Test Conditions) |
| --- | --- |
| Refrigerators Tier II CEE | ≤ 0.06 V + 1.22 kWh/day |
| Freezers Tier II CEE | ≤ 0.28 V + 0.97 kWh/day |
| Glass Door Reach-In Refrigerator Tier I CEE | ≤ 0.12 V + 3.34 kWh/day |
| Glass Door Reach-In Refrigerator Tier II CEE | ≤ 0.86 V + 2.39 kWh/day |

Products are qualified using ASHRAE standard 117-2002 at 38°F +/- 2°F.

V = Internal volume in ft3

AV = Adjusted volume = (1.63 x freezer volume in ft3) + refrigerator volume in ft3

Cost Effectiveness Modeling Measure Data For Solid Door Reach-in Refrigerators and Freezers

Refrigerators and Freezers that meet the CEE Tier II specification results are about 33 percent more efficient than Tier I and provide savings of approximately  445-3,303 kWh, this equates to a savings of $55-$429 annually.

This programs Incentives and savings calculations for the solid door reach-in refrigerators and freezers are based on CEE's Tier II specification using the new California title 20 specifications as the baseline.

Performance data based on ASHRAE Standard 117 2002 at a temperature of 38°F for refrigerators and 0°F for freezers+-2. The nominal size is based on standard widths. Each standard width may include multiple (French style) doors and/or multiple sections.

Estimated annual energy use is based on 365 days per year and the daily energy consumption for typical units as determined by applying ASHRAE 117 2002. Unitized cost effectiveness determinants are summarized in Tables 8.2.1 and 8.2.2. All estimates for energy cost of operation assume an electric rate of $0.13 per kWh. It has been assumed that the appliance’s probable contribution to the building’s peak demand is the average demand for the appliance. This is useful because the probability of an appliance drawing its average rate during the period that the building peak is set is significantly higher than for any other input rate for that appliance.

### Cost Effectiveness Modeling Measure Data for Glass Door Reach-in Refrigerators

Glass Door Refrigerators that meet the CEE Tier I specification are about the top 25 percent of performing models in the market while models that meet the Tier II specification are 28 percent more efficient than Tier I models. Tier I models will provide savings of approximately 445-3,303 kWh while Tier II models will provide savings of 1,000-3000 kWh, this can equate to energy cost savings of $55-$429 annually.

This programs Incentives and savings calculations for the Glass door reach-in refrigerators are based on CEC January 1, 2006 requirements as the baseline. Performance data is based on ASHRAE Standard 117-2002 at a temperature of 38°F+-2. The nominal size is based on standard widths. Each standard width may include multiple doors and/or multiple sections. Estimated annual energy use is based on 365 days per year and the daily energy consumption for typical units as determined by applying ASHRAE 117-2002. Unitized cost effectiveness determinants for glass door reach-in refrigerators are summarized in Tables 8.2.3 and 8.2.4. All  estimates for energy cost of operation assume an electric rate of  $0.13 per kWh. It has been assumed that the appliance’s probable contribution to the building’s peak demand is the average demand for the appliance. This is useful because the probability of an appliance drawing its average rate during the period that the building peak is set is significantly higher than for any other input rate for that appliance.

Table 8.2.1 Commercial Solid Door Reach-in Refrigerator Cost Effectiveness Example

| Description | Solid-Door Refrigerator (CEE Tier II) | Solid-Door Refrigerator (CEE Tier II) | Solid-Door Refrigerator (CEE Tier II) | Solid-Door Refrigerator (CEE Tier II) |
| --- | --- | --- | --- | --- |
| Nominal Sizeb | 1-Door | 1-Door | 2-Door | 3-Door |
| Total Volume (ft3) | < 19 | 19-30 | 31-60 | 61-90 |
| Average Volume used in Energy Calculations | 10 | 24 | 44 | 72 |
| Baseline d | 3.04 | 4.44 | 6.44 | 9.24 |
| Incentive Threshold e | 1.82 | 2.66 | 3.86 | 5.54 |
| Estimated Annual Energy for Baseline f | 1,110 | 1,621 | 2,351 | 3,373 |
| Estimated Annual Energy for EE Unit f | 664 | 971 | 1,409 | 2,022 |
| Estimated Annual Energy Saving g | 445 | 650 | 942 | 1,351 |
| Estimated Annual Energy Cost Saving g | $58 | $84 | $122 | $176 |
| Probable Demand Reduction h | 0.05 | 0.07 | 0.11 | 0.15 |
| Estimated Incremental Cost | SEE APPENDIX A |  |  |  |
| Estimated Useful Life (EUL)f | 12 | 12 | 12 | 12 |

Table 8.2.2 Commercial Solid Door Reach-in Freezer Cost Effectiveness Example

| Description | Solid-Door Freezer (CEE Tier II) | Solid-Door Freezer (CEE Tier II) | Solid-Door Freezer (CEE Tier II) | Solid-Door Freezer (CEE Tier II) |
| --- | --- | --- | --- | --- |
| Nominal Sizea | 1-Door | 1-Door | 2-Door | 3-Door |
| Total Volume (ft3) | < 19 | 19-30 | 31-60 | 61-90 |
| Average Volume used in Energy Calculations | 10 | 24 | 44 | 72 |
| Baseline b | 5.38 | 10.98 | 18.98 | 30.18 |
| Incentive Threshold c | 3.77 | 7.69 | 13.29 | 21.13 |
| Estimated Annual Energy for Baseline d | 1,964 | 4,008 | 6,928 | 1,1016 |
| Estimated Annual Energy for EE Unit d | 1,376 | 2,807 | 4,851 | 7,712 |
| Probable Demand Reduction e | 0.07 | 0.14 | 0.24 | 0.38 |
| Estimated Annual Energy Saving | 588 | 1201 | 2077 | 3303 |
| Estimated Annual Energy Cost Saving | $76 | $156 | $270 | $429 |
| Estimated Incremental Cost | SEE APPENDIX A |  |  |  |
| Estimated Useful Life (EUL)f | 12 | 12 | 12 | 12 |

Table 8.2.3 Glass Door Reach-in Refrigerator Tier I Cost Effectiveness Example

| Description | Glass Door Refrigerator CEE Tier I | Glass Door Refrigerator CEE Tier I | Glass Door Refrigerator CEE Tier I | Glass Door Refrigerator CEE Tier I |
| --- | --- | --- | --- | --- |
| Nominal Sizeb | 1-Door | 1-Door | 2-Door | 3-Door |
| Total Volume (ft3) | < 19 | 19-30 | 31-60 | 61-90 |
| Average Volume used in Energy Calculations | 10 | 24 | 44 | 72 |
| Baseline d | 6.49 | 8.90 | 12.34 | 17.15 |
| Incentive Threshold e | 4.54 | 6.22 | 8.62 | 11.98 |
| Estimated Annual Energy for Baseline f | 2,369 | 3,248 | 4,503 | 6,261 |
| Estimated Annual Energy for EE Unit f | 1,657 | 2,270 | 3,146 | 4,373 |
| Probable Demand Reduction h | 0.08 | 0.11 | 0.15 | 0.22 |
| Estimated Annual Energy Saving g | 712 | 977 | 1,357 | 1,889 |
| Estimated Annual Energy Cost Saving g | $93 | $127 | $176 | $246 |
| Estimated Incremental Costg | SEE APPENDIX A |  |  |  |
| Estimated Useful Life (EUL) h | 12 | 12 | 12 | 12 |

Table 8.2.4 Glass Door Reach-in Refrigerator Tier II Cost Effectiveness Example

| Description | Glass Door Refrigerator CEE Tier II | Glass Door Refrigerator CEE Tier II | Glass Door Refrigerator CEE Tier II | Glass Door Refrigerator CEE Tier II |
| --- | --- | --- | --- | --- |
| Nominal Sizea | 1-Door | 1-Door | 2-Door | 3-Door |
| Total Volume (ft3) | < 19 | 19-30 | 31-60 | 61-90 |
| Average Volume used in Energy Calculations | 10 | 24 | 44 | 72 |
| Baseline b | 6.49 | 8.90 | 12.34 | 17.15 |
| Incentive Threshold c | 3.25 | 4.45 | 6.17 | 8.58 |
| Estimated Annual Energy for Baseline d | 2,369 | 3,248 | 4,503 | 6,261 |
| Estimated Annual Energy for EE Unit d | 1,186 | 1,626 | 2,254 | 3,132 |
| Probable Demand Reduction e | 0.14 | 0.19 | 0.26 | 0.36 |
| Estimated Annual Energy Saving d | 1,183 | 1,622 | 2,250 | 3,129 |
| Estimated Annual Energy Cost Saving f | $154 | $211 | $292 | $407 |
| Estimated Incremental Cost | SEE APPENDIX A |  |  |  |
| Estimated Useful Life (EUL)g | 12 | 12 | 12 | 12 |

a The nominal size is based on standard widths. Each standard width may include multiple doors and/or multiple sections.

b Based on CEC 2005 Appliance Efficiency Regulations, effective August 1, 2004, and representation of installed base.

cTier I levels are based on ENERGY STAR® specifications for commercial solid door refrigerators and freezers. Tier II levels are based on CEE Tier II specifications for commercial solid door refrigerators and freezers.

d Based on 365 days per year and the daily energy use for typical units as determined by applying ASHRAE 117-2002.

e It has been assumed that the appliance’s probable contribution to the building’s peak demand is the average demand for the appliance. This is useful because the probability of an appliance drawing its average rate during the period that the building peak is set is significantly higher than for any other input rate for that appliance.

f  Assumes Electrical Energy Cost of $0.13 per kWh

g  The estimated useful life is based on DEER estimates for food service equipment and filed in the Energy Efficiency Policy Manual Table 4.1.

# Solid Door Reach-In Refrigerator-Commercial

APPENDIX A

| Make Energy Efficient CEE Tier II |  | Model | Sections | FT3 | Cost($)* |
| --- | --- | --- | --- | --- | --- |
| TeknoGard |  | TST-11R-MICRO | 1 | 11.20 | $3,250 |
| Electrolux Home Products |  | 726356 | 1 | 19.86 | $5,800 |
| Continental Refrigerator |  | 2R | 2 | 49.19 | $5,222 |
| Continental Refrigerator |  | 2RE | 2 | 54.54 | $5,746 |
| McCall Refrigeration |  | SXI-2C | 2 | 48.50 | $11,500 |
| Turbo Air Inc. |  | MSR-49NM | 2 | 49.00 | $5,046 |
| Continental Refrigerator |  | 3R | 3 | 72.19 | $7,040 |
| Continental Refrigerator |  | 3RE | 3 | 79.71 | $7,905 |
| Continental Refrigerator |  | DL3RE | 3 | 81.78 | $9,269 |
| Turbo Air Inc. |  | TSR-72SD | 3 | 72.00 | $7,715 |
| Victory Refrigeration |  | RA-3D-S7 | 3 | 70.10 | $8,750 |
| Victory Refrigeration |  | RA-3D-S7-HD | 3 | 70.10 | $9,100 |
| Victory Refrigeration |  | RS-3D-S7 | 3 | 70.10 | $14,980 |
| Victory Refrigeration |  | RS-3D-S7-HD | 3 | 70.10 | $15,450 |
| Victory Refrigeration |  | RSA-3D-S7 | 3 | 70.10 | $12,960 |
| Victory Refrigeration |  | RSA-3D-S7-HD | 3 | 70.10 | $13,440 |
| Victory Refrigeration |  | VR-3 | 3 | 68.70 | $6,440 |
| Victory Refrigeration |  | VSR-3 | 3 | 68.70 | $6,870 |
| Average Cost of CEE Tier II 1-Section <19 ft3 |  |  |  |  | $3,250 |
| Average Cost of CEE Tier II 1-Section |  |  |  |  | $5,800 |
| Average Cost of CEE Tier II 2-Section |  |  |  |  | $6,879 |
| Average Cost of CEE Tier II 3-Section |  |  |  |  | $9,993 |
| Make Baseline ENERGY STAR® | Model |  | Sections | FT3 | Cost($)* |
| True | T-19 |  | 1 | 17.60 | $3,033 |
| True | T-23 |  | 1 | 20.70 | $3,879 |
| True | TS-23 |  | 1 | 22.60 | $4,072 |
| True | T-23-2 |  | 2 | 22.50 | $4,305 |
| True | T-35 |  | 2 | 33.10 | $4,670 |
| True | T-49 |  | 2 | 44.00 | $5,161 |
| True | TBB-2 |  | 2 | 19.60 | $3,637 |
| True | TBB-2-S |  | 2 | 19.60 | $4,333 |
| True | TS-23-2 |  | 2 | 22.50 | $4,522 |
| True | TS-49 |  | 2 | 44.00 | $5,429 |
| True | T-72 |  | 3 | 63.10 | $6,971 |
| True | TS-72 |  | 3 | 63.10 | $7,318 |
| Average Cost of Baseline ENERGY STAR® 1-Section <19 ft3 |  |  |  |  | $3,033 |
| Average Cost of Baseline ENERGY STAR® 1-Section |  |  |  |  | $3,976 |
| Average Cost of Baseline ENERGY STAR® 2-Section |  |  |  |  | $4,580 |
| Average Cost of Baseline ENERGY STAR® 3-Section |  |  |  |  | $7,145 |
| 1-Section <19 ft3 | List Price Average Incremental Cost Difference |  |  |  | $217 |
| 1-Section | List Price Average Incremental Cost Difference |  |  |  | $1,825 |
| 2-Section | List Price Average Incremental Cost Difference |  |  |  | $2,299 |
| 3-Section | List Price Average Incremental Cost Difference |  |  |  | $2,849 |
| *Costs taken from published manufacturers list prices |  |  |  |  |  |

# Solid Door Reach-In Freezers-Commercial

APPENDIX A

| Make Energy Efficient CEE Tier II | Model | Sections | FT3 | Cost($)* |
| --- | --- | --- | --- | --- |
| List Price Estimate |  | 1 | 12.00 | $6,629 |
| List Price Estimate |  | 1 | 24.00 | $6,682 |
| Beverage Air | EF48-1AHS | 2 | 46.20 | $7,409 |
| Beverage Air | EF48-1AS | 2 | 46.20 | $7,030 |
| Beverage Air | KF48-1AS | 2 | 49.00 | $6,717 |
| Continental Refrigeration | 2F | 2 | 49.19 | $6,674 |
| Continental Refrigeration | 2FE | 2 | 54.54 | $7,419 |
| Continental Refrigeration | DL2F | 2 | 50.47 | $7,657 |
| Continental Refrigeration | DL2FE | 2 | 55.96 | $7,950 |
| Hoshizaki America, Inc. | FH2-AAC | 2 | 46.10 | $7,300 |
| Traulsen & Co., Inc. | ALT232WUT-FHS | 2 | 51.60 | $12,391 |
| Traulsen & Co., Inc. | ALT232WUT-HHS | 2 | 51.60 | $12,769 |
| Traulsen & Co., Inc. | RLT232WUT-FHS | 2 | 51.60 | $13,838 |
| Traulsen & Co., Inc. | RLT232WUT-HHS | 2 | 51.60 | $14,269 |
| Turbo Air Inc. | MSF-49NM | 2 | 49.00 | $6,384 |
| Turbo Air Inc. | TSF-49SD | 2 | 49.00 | $6,792 |
| Victory Refrigeration | FA-2D-S7 | 2 | 46.50 | $7,830 |
| Victory Refrigeration | FA-2D-S7-HD | 2 | 46.50 | $8,070 |
| Victory Refrigeration | FS-2D-S7 | 2 | 46.50 | $13,200 |
| Victory Refrigeration | FS-2D-S7-HD | 2 | 46.50 | $13,560 |
| Victory Refrigeration | FSA-2D-S7 | 2 | 46.50 | $11,660 |
| Victory Refrigeration | FSA-2D-S7-HD | 2 | 46.50 | $12,020 |
| Beverage Air | EF74-5AHS | 3 | 69.50 | $10,115 |
| Beverage Air | EF74-5AS | 3 | 69.50 | $9,546 |
| Beverage Air | PF3-5AHS-XDX | 3 | 64.40 | $17,062 |
| Beverage Air | PF3-5AS-XDX | 3 | 64.40 | $15,675 |
| Nor-Lake Incorporated | NF803SSS/8 | 3 | 80.00 | $8,565 |
| Nor-Lake Incorporated | NF806SSS/8 | 3 | 80.00 | $10,392 |
| Turbo Air Inc. | TSF-72SD | 3 | 72.00 | $9,618 |
| Victory Refrigeration | FS-3D-S7-EW | 3 | 79.60 | $19,580 |
| Victory Refrigeration | FS-3D-S7-EW-HD | 3 | 79.60 | $20,170 |
| Victory Refrigeration | FSA-3D-S7-EW | 3 | 79.60 | $17,220 |
| Victory Refrigeration | FSA-3D-SA7-EW-HD | 3 | 79.60 | $17,680 |
| Average Cost of CEE Tier II Freezer 1-Section <19 Ft3 |  |  |  | $6,871 |
| Average Cost of CEE Tier II Freezer 1-Section |  |  |  | $6,927 |
| Average Cost of CEE Tier II Freezer 2-Section |  |  |  | $9,547 |
| Average Cost of CEE Tier II Freezer 3-Section |  |  |  | $14,148 |

Solid Door Reach-In Freezers-Commercial

| Make Baseline ENERGY STAR® | Model |  | Sections | FT3 |  | Cost($)* |
| --- | --- | --- | --- | --- | --- | --- |
| Continental Refrigeration | DL1FSE |  | 1 | 16.22 |  | $5,888 |
| Beverage Air | KF24-1AS |  | 1 | 23.00 |  | $4,886 |
| Beverage Air | PF1-1AS-XDX |  | 1 | 22.10 |  | $7,695 |
| Continental Refrigeration | 1F |  | 1 | 20.05 |  | $4,913 |
| Continental Refrigeration | 1FE |  | 1 | 21.00 |  | $5,400 |
| Continental Refrigeration | DL1F |  | 1 | 20.57 |  | $5,610 |
| Continental Refrigeration | DL1FE |  | 1 | 22.00 |  | $5,837 |
| Continental Refrigeration | DL1FX |  | 1 | 31.72 |  | $7,110 |
| Hoshizaki America, Inc. | FH1-AAC |  | 1 | 20.85 |  | $5,490 |
| Hoshizaki America, Inc. | FH1-SSB |  | 1 | 20.60 |  | $7,510 |
| Traulsen & Co., Inc. | G12000 |  | 1 | 24.20 |  | $6,054 |
| Traulsen & Co., Inc. | G12001 |  | 1 | 24.20 |  | $6,054 |
| Traulsen & Co., Inc. | G12010 |  | 1 | 24.20 |  | $5,819 |
| Make Baseline ENERGY STAR® | Model |  | Sections | FT3 |  | Cost($)* |
| Traulsen & Co., Inc. | G12011 |  | 1 | 24.20 |  | $5,819 |
| Victory Refrigeration | VF-1 |  | 1 | 20.50 |  | $4,780 |
| Victory Refrigeration | VF-1-HD |  | 1 | 20.50 |  | $5,420 |
| Victory Refrigeration | VF-2 |  | 1 | 44.90 |  | $6,570 |
| Beverage Air | PF2-1AS-XDX |  | 2 | 48.20 |  | $11,717 |
| Continental Refrigeration | 2FE-PT |  | 2 | 54.54 |  | $9,952 |
| Continental Refrigeration | 2F-PT |  | 2 | 49.19 |  | $9,579 |
| Continental Refrigeration | 2FSE |  | 2 | 30.32 |  | $6,255 |
| Continental Refrigeration | DL1FX-PT |  | 2 | 31.72 |  | $9,785 |
| Continental Refrigeration | DL2FSE |  | 2 | 31.72 |  | $6,633 |
| Hoshizaki America, Inc. | FH2-AAC-HD |  | 2 | 46.10 |  | $7,790 |
| Hoshizaki America, Inc. | FH2-SSB |  | 2 | 45.05 |  | $9,630 |
| Hoshizaki America, Inc. | FH2-SSB-HD |  | 2 | 45.05 |  | $10,310 |
| Hoshizaki America, Inc. | HUF68A |  | 2 | 20.20 |  | $7,120 |
| Hoshizaki America, Inc. | HWF68A |  | 2 | 20.20 |  | $7,400 |
| Traulsen & Co., Inc. | G22000 |  | 2 | 46.00 |  | $8,056 |
| Traulsen & Co., Inc. | G22001 |  | 2 | 46.00 |  | $8,056 |
| Traulsen & Co., Inc. | G22002 |  | 2 | 46.00 |  | $8,056 |
| Traulsen & Co., Inc. | G22003 |  | 2 | 46.00 |  | $8,056 |
| Victory Refrigeration | VF-2-HD |  | 2 | 44.90 |  | $7,280 |
| Beverage Air | KF74-5AS |  | 3 | 72.00 |  | $9,376 |
| Continental Refrigeration | 3F |  | 3 | 72.19 |  | $9,087 |
| Continental Refrigeration | 3FE |  | 3 | 73.00 |  | $10,333 |
| Continental Refrigeration | 3FE-PT |  | 3 | 79.71 |  | $13,461 |
| Continental Refrigeration | 3F-PT |  | 3 | 72.19 |  | $12,953 |
| Continental Refrigeration | 3FS |  | 3 | 57.75 |  | $12,235 |
| Continental Refrigeration | DL3F |  | 3 | 74.06 |  | $10,736 |
| Continental Refrigeration | DL3FE |  | 3 | 81.78 |  | $11,155 |
| Continental Refrigeration | DL3FE-PT |  | 3 | 81.78 |  | $16,339 |
| Continental Refrigeration | DL3F-PT |  | 3 | 74.06 |  | $15,723 |
| Traulsen & Co., Inc. | G31010 |  | 3 | 69.10 |  | $10,834 |
| Traulsen & Co., Inc. | G31011 |  | 3 | 69.10 |  | $10,834 |
| Traulsen & Co., Inc. | G31012 |  | 3 | 69.10 |  | $10,834 |
| Traulsen & Co., Inc. | G31013 |  | 3 | 69.10 |  | $10,834 |
| Traulsen & Co., Inc. | G31300 |  | 3 | 69.10 |  | $11,124 |
| Traulsen & Co., Inc. | G31301 |  | 3 | 69.10 |  | $11,124 |
| Traulsen & Co., Inc. | G31302 |  | 3 | 69.10 |  | $11,124 |
| Traulsen & Co., Inc. | G31303 |  | 3 | 69.10 |  | $11,124 |
| Traulsen & Co., Inc. | G31310 |  | 3 | 69.10 |  | $10,749 |
| Traulsen & Co., Inc. | G31311 |  | 3 | 69.10 |  | $10,749 |
| Victory Refrigeration | FA-3D-S7-HD |  | 3 | 70.10 |  | $11,450 |
| Victory Refrigeration | FS-3D-S7-HD |  | 3 | 70.10 |  | $18,750 |
| Victory Refrigeration | FSA-3D-S7-HD |  | 3 | 70.10 |  | $16,190 |
| Victory Refrigeration | VF-3 |  | 3 | 68.70 |  | $8,350 |
| Victory Refrigeration | VF-3-HD |  | 3 | 68.70 |  | $10,120 |
| Average Cost of Baseline ENERGY STAR® Freezer 1-Section <19 ft3 |  |  |  |  |  | $5,888 |
| Average Cost of Baseline ENERGY STAR® Freezer 1-Section |  |  |  |  |  | $5,935 |
| Average Cost of Baseline ENERGY STAR® Freezer 2-Section |  |  |  |  |  | $8,480 |
| Average Cost of Baseline ENERGY STAR® Freezer 3-Section |  |  |  |  |  | $11,824 |
| 1-Section <19 ft3** |  | List Price Average Incremental Cost Difference |  |  | $741 |  |
| 1-Section** |  | List Price Average Incremental Cost Difference |  |  | $747 |  |
| 2-Section |  | List Price Average Incremental Cost Difference |  |  | $1,067 |  |
| 3-Section |  | List Price Average Incremental Cost Difference |  |  | $2,324 |  |
| *Costs taken from published manufacturers lists price |  |  |  |  |  |  |
| **List Price was not available for Tier II Energy Efficient product. A ratio of baseline case to IMC for 2-Section freezer was applied to baseline case cost of this configuration as an IMC estimate |  |  |  |  |  |  |

# Glass Door Reach-In Refrigerators-Commercial

APPENDIX A

| Make Energy Efficient CEE Tier II | Model | Sections | FT3 | Cost($)* |
| --- | --- | --- | --- | --- |
| Beverage Air | UC85-1** | 1 | 2.40 | $1,535 |
| Traulsen-(Hobart) | UHT48 (CU48) | 1 | 13.10 | $4,310 |
| Traulsen-(Hobart) | UHT60 (CU60) | 1 | 16.70 | $4,713 |
| List Price Estimate |  | 1 | 24.00 | $6,963 |
| Victory Refrigeration | RA-2D-S7-GD | 2 | 46.50 | $7,830 |
| List Price Estimate |  | 3 | 70.00 | $8,015 |
| Average Cost of CEE Tier II Glass Door Refrigerator 1-Section <19 ft3 |  |  |  | $3,519 |
| Average Cost of CEE Tier II Glass Door Refrigerator 1-Section |  |  |  | $5,816 |
| Average Cost of CEE Tier II Glass Door Refrigerator 2-Section |  |  |  | $7,830 |
| Average Cost of CEE Tier II Glass Door Refrigerator 3-Section |  |  |  | $8,594 |
| Make Energy Efficient CEE Tier I | Model | Sections | FT3 | Cost($)* |
| Turbo Air | TGM-5R | 1 | 4.70 | $1,131 |
| Beverage Air | CR5-1W-G | 1 | 4.81 | $2,907 |
| True | GDM-10PT | 1 | 8.10 | $3,246 |
| True | GDM-12FC | 1 | 10.10 | $2,851 |
| Hussmann Corp | BCH10 | 1 | 12.39 | $2,366 |
| Turbo Air | MSR-23G-1 | 1 | 20.00 | $4,153 |
| True | GDM-23RF | 1 | 20.10 | $3,989 |
| Victory Refrigeration | VM-1 | 1 | 21.00 | $3,500 |
| Victory Refrigeration | RA-1D-S7-GD | 1 | 21.50 | $6,070 |
| True | TS23G | 1 | 22.60 | $4,655 |
| True | GDM-26 | 1 | 23.00 | $3,092 |
| Randell Manufacturing | 2011 | 1 | 23.10 | $5,475 |
| Beverage Air | DMC*4** | 2 | 28.30 | $7,426 |
| Beverage Air | ER48-1A*g | 2 | 46.70 | $7,426 |
| Victory Refrigeration | RA-2D-S7-GD | 2 | 46.50 | $7,830 |
| Beverage Air | MT72-***-55****** | 3 | 72.00 | $6,398 |
| Average Cost of CEE Tier I Glass Door Refrigerator 1-Section <19 ft3 |  |  |  | $2,500 |
| Average Cost of CEE Tier I Glass Door Refrigerator 1-Section |  |  |  | $4,795 |
| Average Cost of CEE Tier I Glass Door Refrigerator 2-Section |  |  |  | $7,830 |
| Average Cost of CEE Tier I Glass Door Refrigerator 3-Section |  |  |  | $6,398 |

| Make Baseline | Model |  | Sections | FT3 | Cost($)* |  |
| --- | --- | --- | --- | --- | --- | --- |
| Beverage Air | CT96 |  | 1 | 5.83 | $2,139 |  |
| Turbo Air | TGM-11RV |  | 1 | 10.00 | $1,965 |  |
| Beverage Air | MT12-1-B |  | 1 | 10.38 | $2,162 |  |
| Turbo Air | TGM-14R |  | 1 | 14.00 | $2,165 |  |
| True | T-19G |  | 1 | 19.00 | $3,686 |  |
| True | TS-23G |  | 1 | 23.00 | $4,655 |  |
| True | TS-49G |  | 2 | 49.00 | $6,401 |  |
| True | TSD-33G |  | 2 | 33.00 | $5,474 |  |
| True | TSD-47G |  | 2 | 47.00 | $6,097 |  |
| Cornelius | COR67VMST |  | 3 | 67.00 | $5,815 |  |
| Hussmann | ARV-1050 |  | 3 | 69.00 | $6,450 |  |
| Average Cost of Baseline Glass Door Refrigerator 1-Section <19 ft3 |  |  |  |  | $2,108 |  |
| Average Cost of Baseline Glass Door Refrigerator 1-Section |  |  |  |  | $4,171 |  |
| Average Cost of Baseline Glass Door Refrigerator 2-Section |  |  |  |  | $5,991 |  |
| Average Cost of Baseline Glass Door Refrigerator 3-Section |  |  |  |  | $6,133 |  |
| Tier II to Baseline 1-Section <19 ft3 |  | List Price Average Incremental Cost Difference |  |  |  | $1,412 |
| Tier II to Baseline 1-Section** |  | List Price Average Incremental Cost Difference |  |  |  | $2,793 |
| Tier II to Baseline 2-Section |  | List Price Average Incremental Cost Difference |  |  |  | $1,839 |
| Tier II to Baseline 3-Section** |  | List Price Average Incremental Cost Difference |  |  |  | $1,883 |
| Tier I to Baseline 1-Section <19 ft3 |  | List Price Average Incremental Cost Difference |  |  |  | $392 |
| Tier I to Baseline 1-Section |  | List Price Average Incremental Cost Difference |  |  |  | $624 |
| Tier I to Baseline 2-Section |  | List Price Average Incremental Cost Difference |  |  |  | $1,839 |
| Tier I to Baseline 3-Section |  | List Price Average Incremental Cost Difference |  |  |  | $266 |
| *Costs taken from published manufacturers list prices |  |  |  |  |  |  |
| ** List Price was not available for Tier II Energy Efficient product. A ratio of base case to IMC for 1-Section 20 ft3 was applied to baseline case cost of this configuration as an IMC estimate |  |  |  |  |  |  |

American Society for Testing and Materials. Standard Test Method for the Performance of Open Deep Fat Fryers. ASTM Designation F1361-05, in Annual Book of ASTM Standards, West Conshohocken, PA.

American Society for Testing and Materials. Standard Test Method for the Performance of Griddles. ASTM Designation F1275 99, in Annual Book of ASTM Standards, West Conshohocken, PA.

American Society for Testing and Materials. 2005. Standard Test Method for the Performance of Steam Cookers. ASTM Designation F1484-05, in Annual Book of ASTM Standards, West Conshohocken, PA.

A supplement to Restaurant Business Inc., 1995. Foodservice Equipment 1000 for NAFEM. The Baking Boom, p.53 54.

American Society for Testing and Materials. 1999. Standard Test Method for the Performance of Convection Ovens. ASTM Designation F1496 99, in Annual Book of ASTM Standards, West Conshohocken, PA.

A supplement to Restaurant Business Inc., 1995. Foodservice Equipment 1000 for NAFEM. The Baking Boom, p.53-54.

American Society for Testing and Materials. Standard Test Method for the Performance of Convection Ovens. ASTM Designation F1639-05. in Annual Book of ASTM Standards, West Conshohocken, PA.

CEE Commercial Ice Machine Program FAQ

Source: Energy Savings Potential for Commercial Refrigeration Equipment, ADL 1996.

Source: Energy Savings Potential for Commercial Refrigeration Equipment, ADL 1996.

Source: EPA 2003.

Source: Energy Savings Potential for Commercial Refrigeration Equipment, ADL 1996.

Program fact sheet for commercial refrigeration listed on www.CEE1.org Web site.